EE4620L/EE6620L/CEG4324L/CEG6324L

DIGITAL INTEGRATED CIRCUIT DESIGN LAB

Project

By

Current, Logan

UID: U01019510

Email: Current.22@wright.edu

Submission date: 04/18/2024

“I have neither given nor received aid on this assignment, nor have I observed any violation of the Honor code”

Signature :

Date : 04/18/2024

Table of Contents

1. Aim/Objective (p.g. 3)

2. 12-bit pipeline Arch & WPD Calculation (p.g 3)

3. VHDL Code & TB Code (p.g. 4)

4. Simulation Waveform (p.g. 21)

5. Post-synthesis Schematic Diagram (p.g. 22)

6.  Hardware Report (p.g. 24)

7. Compare Hardware & Clock Speed (p.g. 26)

8. Conclusion (p.g. 26)

List of Figures

2. 12-bit pipelining Arch & WPD Calculation: 2 figures showing how the pipelining will look for each case conceptually

4. Simulation Waveform: 2 images showing the waveform for each of the cases

5. Post-synthesis Schematic Diagram: 2 images showing the schematic diagram

6. Hardware report: 2 images portraying the hardware generated from the code and compiler

- AIM / OBJECTIVE:

The aim of this lab was to explore and design the concept of pipelining as well as implementing it in the Vivado environment.

- 12 -bit pipeline Arch & WPD Calculation:

Case 1:

WPD = (6 * 15ns) + (2 * 5ns) = 100ns

Case 2:

WPD = (4 * 15ns) + (2 * 5ns) = 70ns

- VHDL Code & TB Code:

VHDL Code:

Case 1 :

----------------------------------------------------------------------------------

---REGISTER

library ieee;

use ieee.std_logic_1164.all;

entity REG is

port(D, CLK, RSTn: in std_logic;

Q: out std_logic);

end entity;

architecture behaviour of REG is

begin

process(CLK)

begin

if (rising_edge(CLK)) then

if(RSTn = '1') then

Q <= '0'

--pragma_sythesis_off

after 10ns

--pragma_sythesis_on

;

else

Q <= D

--pragma_sythesis_off

after 10ns

--pragma_sythesis_on

;

end if;

end if;

end process;

end architecture;

----------------------------------------------------------------------------------

---FULL ADDER

library ieee;

use ieee.std_logic_1164.all;

entity FA is

port (A, B, Ci: IN STD_LOGIC ;

S, Co: OUT STD_LOGIC ) ;

end entity;

architecture behaviour of FA is

begin

S  <= A XOR B XOR Ci

--pragma_sythesis_off

after 10ns

--pragma_sythesis_on

;

Co <= (A AND B) OR (Ci AND A) OR (Ci AND B)

--pragma_sythesis_off

after 15ns

--pragma_sythesis_on

;

end architecture;

----------------------------------------------------------------------------------

---PIPELINED ADDER

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity pipelined_adder is

generic(N :integer := 12); --4 bit pipelined adder is given here as an example

Port   (A  : in STD_LOGIC_VECTOR (N-1 downto 0);

B  : in STD_LOGIC_VECTOR (N-1 downto 0);

Ci : in STD_LOGIC;

S  : out STD_LOGIC_VECTOR (N downto 0);

CLK, RSTn : in STD_LOGIC );

end pipelined_adder;

architecture Behavioral of pipelined_adder is

--- component declaration

component FA

PORT (A, B, Ci: IN STD_LOGIC ;

S, Co: OUT STD_LOGIC ) ;

end component;

component REG

port(D, CLK, RSTn: in std_logic;

Q: out std_logic);

end component;

--- signal declaration example is given below.

--- You'll need to modify signals and their vector sizes for your pipeline designs

signal Co       : std_logic;    --use this signal as your last carry out signal

signal tmp_cout : std_logic_vector(12 downto 0);

signal tmp_S0   : std_logic_vector(1 downto 0);

signal tmp_S1   : std_logic_vector(1 downto 0);

signal tmp_S2   : std_logic_vector(1 downto 0);

signal tmp_S3   : std_logic_vector(1 downto 0);

signal tmp_S4   : std_logic_vector(1 downto 0);

signal tmp_S5   : std_logic_vector(1 downto 0);

signal tmp_S6   : std_logic;

signal tmp_S7   : std_logic;

signal tmp_S8   : std_logic;

signal tmp_S9   : std_logic;

signal tmp_S10   : std_logic;

signal tmp_S11   : std_logic;

signal tmp_A6 : std_logic;

signal tmp_B6 : std_logic;

signal tmp_A7 : std_logic;

signal tmp_B7 : std_logic;

signal tmp_A8 : std_logic;

signal tmp_B8 : std_logic;

signal tmp_A9 : std_logic;

signal tmp_B9 : std_logic;

signal tmp_A10 : std_logic;

signal tmp_B10 : std_logic;

signal tmp_A11 : std_logic;

signal tmp_B11 : std_logic;

--- add necessary signals here

begin

--- 1st Full Adder

FA_0    : FA port map(A(0), B(0), Ci, tmp_S0(0), tmp_cout(0));

reg_s0_0: REG port map(tmp_S0(0), CLK, RSTn, tmp_S0(1));

reg_s0_1: REG port map(tmp_S0(1), CLK, RSTn, S(0));

--- 2nd Full Adder

FA_1    : FA port map(A(1), B(1), tmp_cout(0), tmp_S1(0), tmp_cout(1));

reg_s1_0: REG port map(tmp_S1(0), CLK, RSTn, tmp_S1(1));

reg_s1_1: REG port map(tmp_S1(1), CLK, RSTn, S(1));

---3rd Full Adder

FA_2: FA port map(A(2), B(2), tmp_cout(1), tmp_S2(0), tmp_cout(2));

reg_s2_0: REG port map(tmp_S2(0), CLK, RSTn, tmp_S2(1));

reg_s2_1: REG port map(tmp_S2(1), CLK, RSTn, S(2));

--- 4th Full Adder

FA_3: FA port map(A(3), B(3), tmp_cout(2), tmp_S3(0), tmp_cout(3));

reg_s3_0: REG port map(tmp_S3(0), CLK, RSTn, tmp_S3(1));

reg_s3_1: REG port map(tmp_S3(1), CLK, RSTn, S(3));

--- 5th Full Adder

FA_4: FA port map(A(4), B(4), tmp_cout(3), tmp_S4(0), tmp_cout(4));

reg_s4_0: REG port map(tmp_S4(0), CLK, RSTn, tmp_S4(1));

reg_s4_1: REG port map(tmp_S4(1), CLK, RSTn, S(4));

--- 6th Full Adder

FA_5: FA port map(A(5), B(5), tmp_cout(4), tmp_S5(0), tmp_cout(5));

reg_s5_0: REG port map(tmp_S5(0), CLK, RSTn, tmp_S5(1));

reg_s5_1: REG port map(tmp_S5(1), CLK, RSTn, S(5));

cout_reg_0: REG port map(tmp_cout(5), CLK, RSTn, tmp_cout(6));

--- 7th Full Adder

reg_tmp_A6: REG port map(A(6), CLK, RSTn, tmp_A6);

reg_tmp_B6: REG port map(B(6), CLK, RSTn, tmp_B6);

FA_6: FA port map(tmp_A6, tmp_B6, tmp_cout(6), tmp_S6, tmp_cout(7));

reg_s6: REG port map(tmp_S6, CLK, RSTn, S(6));

--- 8th Full Adder

reg_tmp_A7: REG port map(A(7), CLK, RSTn, tmp_A7);

reg_tmp_B7: REG port map(B(7), CLK, RSTn, tmp_B7);

FA_7: FA port map(tmp_A7, tmp_B7, tmp_cout(7), tmp_S7, tmp_cout(8));

reg_s7: REG port map(tmp_S7, CLK, RSTn, S(7));

--- 9th Full Adder

reg_tmp_A8: REG port map(A(8), CLK, RSTn, tmp_A8);

reg_tmp_B8: REG port map(B(8), CLK, RSTn, tmp_B8);

FA_8: FA port map(tmp_A8, tmp_B8, tmp_cout(8), tmp_S8, tmp_cout(9));

reg_s8: REG port map(tmp_S8, CLK, RSTn, S(8));

--- 10th Full Adder

reg_tmp_A9: REG port map(A(9), CLK, RSTn, tmp_A9);

reg_tmp_B9: REG port map(B(9), CLK, RSTn, tmp_B9);

FA_9: FA port map(tmp_A9, tmp_B9, tmp_cout(9), tmp_S9, tmp_cout(10));

reg_s9: REG port map(tmp_S9, CLK, RSTn, S(9));

--- 11th Full Adder

reg_tmp_A10: REG port map(A(10), CLK, RSTn, tmp_A10);

reg_tmp_B10: REG port map(B(10), CLK, RSTn, tmp_B10);

FA_10: FA port map(tmp_A10, tmp_B10, tmp_cout(10), tmp_S10, tmp_cout(11));

reg_s10: REG port map(tmp_S10, CLK, RSTn, S(10));

--- 12th Full Adder

reg_tmp_A11: REG port map(A(11), CLK, RSTn, tmp_A11);

reg_tmp_B11: REG port map(B(11), CLK, RSTn, tmp_B11);

FA_11: FA port map(tmp_A11, tmp_B11, tmp_cout(11), tmp_S11, tmp_cout(12));

reg_s11: REG port map(tmp_S11, CLK, RSTn, S(11));

cout_reg_1: REG port map(tmp_cout(12), CLK, RSTn, Co);

S(N)<=Co;

end Behavioral;

Case 2 :

----------------------------------------------------------------------------------

---REGISTER

library ieee;

use ieee.std_logic_1164.all;

entity REG is

port(D, CLK, RSTn: in std_logic;

Q: out std_logic);

end entity;

architecture behaviour of REG is

begin

process(CLK)

begin

if (rising_edge(CLK)) then

if(RSTn = '1') then

Q <= '0'

--pragma_sythesis_off

after 10ns

--pragma_sythesis_on

;

else

Q <= D

--pragma_sythesis_off

after 10ns

--pragma_sythesis_on

;

end if;

end if;

end process;

end architecture;

----------------------------------------------------------------------------------

---FULL ADDER

library ieee;

use ieee.std_logic_1164.all;

entity FA is

port (A, B, Ci: IN STD_LOGIC ;

S, Co: OUT STD_LOGIC ) ;

end entity;

architecture behaviour of FA is

begin

S  <= A XOR B XOR Ci

--pragma_sythesis_off

after 10ns

--pragma_sythesis_on

;

Co <= (A AND B) OR (Ci AND A) OR (Ci AND B)

--pragma_sythesis_off

after 15ns

--pragma_sythesis_on

;

end architecture;

----------------------------------------------------------------------------------

---PIPELINED ADDER

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity pipelined_adder is

generic(N :integer := 12); --4 bit pipelined adder is given here as an example

Port   (A  : in STD_LOGIC_VECTOR (N-1 downto 0);

B  : in STD_LOGIC_VECTOR (N-1 downto 0);

Ci : in STD_LOGIC;

S  : out STD_LOGIC_VECTOR (N downto 0);

CLK, RSTn : in STD_LOGIC );

end pipelined_adder;

architecture Behavioral of pipelined_adder is

--- component declaration

component FA

PORT (A, B, Ci: IN STD_LOGIC ;

S, Co: OUT STD_LOGIC ) ;

end component;

component REG

port(D, CLK, RSTn: in std_logic;

Q: out std_logic);

end component;

--- signal declaration example is given below.

--- You'll need to modify signals and their vector sizes for your pipeline designs

signal Co       : std_logic;    --use this signal as your last carry out signal

signal tmp_cout : std_logic_vector(13 downto 0);

signal tmp_S0   : std_logic_vector(2 downto 0);

signal tmp_S1   : std_logic_vector(2 downto 0);

signal tmp_S2   : std_logic_vector(2 downto 0);

signal tmp_S3   : std_logic_vector(2 downto 0);

signal tmp_S4   : std_logic_vector(1 downto 0);

signal tmp_S5   : std_logic_vector(1 downto 0);

signal tmp_S6   : std_logic_vector(1 downto 0);

signal tmp_S7   : std_logic_vector(1 downto 0);

signal tmp_S8   : std_logic;

signal tmp_S9   : std_logic;

signal tmp_S10   : std_logic;

signal tmp_S11   : std_logic;

signal tmp_A4 : std_logic;

signal tmp_B4 : std_logic;

signal tmp_A5 : std_logic;

signal tmp_B5 : std_logic;

signal tmp_A6 : std_logic;

signal tmp_B6 : std_logic;

signal tmp_A7 : std_logic;

signal tmp_B7 : std_logic;

signal tmp_A8 : std_logic_vector(1 downto 0);

signal tmp_B8 : std_logic_vector(1 downto 0);

signal tmp_A9 : std_logic_vector(1 downto 0);

signal tmp_B9 : std_logic_vector(1 downto 0);

signal tmp_A10 : std_logic_vector(1 downto 0);

signal tmp_B10 : std_logic_vector(1 downto 0);

signal tmp_A11 : std_logic_vector(1 downto 0);

signal tmp_B11 : std_logic_vector(1 downto 0);

--- add necessary signals here

begin

--- 1st Full Adder

FA_0    : FA port map(A(0), B(0), Ci, tmp_S0(0), tmp_cout(0));

reg_s0_0: REG port map(tmp_S0(0), CLK, RSTn, tmp_S0(1));

reg_s0_1: REG port map(tmp_S0(1), CLK, RSTn, tmp_S0(2));

reg_s0_2: REG port map(tmp_S0(2), CLK, RSTn, S(0));

--- 2nd Full Adder

FA_1    : FA port map(A(1), B(1), tmp_cout(0), tmp_S1(0), tmp_cout(1));

reg_s1_0: REG port map(tmp_S1(0), CLK, RSTn, tmp_S1(1));

reg_s1_1: REG port map(tmp_S1(1), CLK, RSTn, tmp_S1(2));

reg_s1_2: REG port map(tmp_S1(2), CLK, RSTn, S(1));

---3rd Full Adder

FA_2: FA port map(A(2), B(2), tmp_cout(1), tmp_S2(0), tmp_cout(2));

reg_s2_0: REG port map(tmp_S2(0), CLK, RSTn, tmp_S2(1));

reg_s2_1: REG port map(tmp_S2(1), CLK, RSTn, tmp_S2(2));

reg_s2_2: REG port map(tmp_S2(2), CLK, RSTn, S(2));

--- 4th Full Adder

FA_3: FA port map(A(3), B(3), tmp_cout(2), tmp_S3(0), tmp_cout(3));

reg_s3_0: REG port map(tmp_S3(0), CLK, RSTn, tmp_S3(1));

reg_s3_1: REG port map(tmp_S3(1), CLK, RSTn, tmp_S3(2));

reg_s3_2: REG port map(tmp_S3(2), CLK, RSTn, S(3));

cout_reg_0: REG port map(tmp_cout(3), CLK, RSTn, tmp_cout(4));

--- 5th Full Adder

reg_tmp_A4: REG port map(A(4), CLK, RSTn, tmp_A4);

reg_tmp_B4: REG port map(B(4), CLK, RSTn, tmp_B4);

FA_4: FA port map(tmp_A4, tmp_B4, tmp_cout(4), tmp_S4(0), tmp_cout(5));

reg_s4_0: REG port map(tmp_S4(0), CLK, RSTn, tmp_S4(1));

reg_s4_1: REG port map(tmp_S4(1), CLK, RSTn, S(4));

--- 6th Full Adder

reg_tmp_A5: REG port map(A(5), CLK, RSTn, tmp_A5);

reg_tmp_B5: REG port map(B(5), CLK, RSTn, tmp_B5);

FA_5: FA port map(tmp_A5, tmp_B5, tmp_cout(5), tmp_S5(0), tmp_cout(6));

reg_s5_0: REG port map(tmp_S5(0), CLK, RSTn, tmp_S5(1));

reg_s5_1: REG port map(tmp_S5(1), CLK, RSTn, S(5));

--- 7th Full Adder

reg_tmp_A6: REG port map(A(6), CLK, RSTn, tmp_A6);

reg_tmp_B6: REG port map(B(6), CLK, RSTn, tmp_B6);

FA_6: FA port map(tmp_A6, tmp_B6, tmp_cout(6), tmp_S6(0), tmp_cout(7));

reg_s6_0: REG port map(tmp_S6(0), CLK, RSTn, tmp_S6(1));

reg_s6_1: REG port map(tmp_S6(1), CLK, RSTn, S(6));

--- 8th Full Adder

reg_tmp_A7: REG port map(A(7), CLK, RSTn, tmp_A7);

reg_tmp_B7: REG port map(B(7), CLK, RSTn, tmp_B7);

FA_7: FA port map(tmp_A7, tmp_B7, tmp_cout(7), tmp_S7(0), tmp_cout(8));

reg_s7_0: REG port map(tmp_S7(0), CLK, RSTn, tmp_S7(1));

reg_s7_1: REG port map(tmp_S7(1), CLK, RSTn, S(7));

cout_reg_1: REG port map(tmp_cout(8), CLK, RSTn, tmp_cout(9));

--- 9th Full Adder

reg_tmp_A8_0: REG port map(A(8), CLK, RSTn, tmp_A8(0));

reg_tmp_B8_0: REG port map(B(8), CLK, RSTn, tmp_B8(0));

reg_tmp_A8_1: REG port map(tmp_A8(0), CLK, RSTn, tmp_A8(1));

reg_tmp_B8_1: REG port map(tmp_B8(0), CLK, RSTn, tmp_B8(1));

FA_8: FA port map(tmp_A8(1), tmp_B8(1), tmp_cout(9), tmp_S8, tmp_cout(10));

reg_s8: REG port map(tmp_S8, CLK, RSTn, S(8));

--- 10th Full Adder

reg_tmp_A9_0: REG port map(A(9), CLK, RSTn, tmp_A9(0));

reg_tmp_B9_0: REG port map(B(9), CLK, RSTn, tmp_B9(0));

reg_tmp_A9_1: REG port map(tmp_A9(0), CLK, RSTn, tmp_A9(1));

reg_tmp_B9_1: REG port map(tmp_B9(0), CLK, RSTn, tmp_B9(1));

FA_9: FA port map(tmp_A9(1), tmp_B9(1), tmp_cout(10), tmp_S9, tmp_cout(11));

reg_s9: REG port map(tmp_S9, CLK, RSTn, S(9));

--- 11th Full Adder

reg_tmp_A10_0: REG port map(A(10), CLK, RSTn, tmp_A10(0));

reg_tmp_B10_0: REG port map(B(10), CLK, RSTn, tmp_B10(0));

reg_tmp_A10_1: REG port map(tmp_A10(0), CLK, RSTn, tmp_A10(1));

reg_tmp_B10_1: REG port map(tmp_B10(0), CLK, RSTn, tmp_B10(1));

FA_10: FA port map(tmp_A10(1), tmp_B10(1), tmp_cout(11), tmp_S10, tmp_cout(12));

reg_s10: REG port map(tmp_S10, CLK, RSTn, S(10));

--- 12th Full Adder

reg_tmp_A11_0: REG port map(A(11), CLK, RSTn, tmp_A11(0));

reg_tmp_B11_0: REG port map(B(11), CLK, RSTn, tmp_B11(0));

reg_tmp_A11_1: REG port map(tmp_A11(0), CLK, RSTn, tmp_A11(1));

reg_tmp_B11_1: REG port map(tmp_B11(0), CLK, RSTn, tmp_B11(1));

FA_11: FA port map(tmp_A11(1), tmp_B11(1), tmp_cout(12), tmp_S11, tmp_cout(13));

reg_s11: REG port map(tmp_S11, CLK, RSTn, S(11));

cout_reg_2: REG port map(tmp_cout(13), CLK, RSTn, Co);

S(N)<=Co;

end Behavioral;

TB Code:

----------------------------------------------------------------------------------

-- Company:

-- Engineer:

--

-- Create Date:

-- Design Name:

-- Module Name: tb_pipelined_adder - Behavioral

-- Project Name:

-- Target Devices:

-- Tool Versions:

-- Description:

--

-- Dependencies:

--

-- Revision:

-- Revision 0.01 - File Created

-- Additional Comments:

--

----------------------------------------------------------------------------------

library IEEE;

use IEEE.STD_LOGIC_1164.ALL;

entity tb_pipelined_adder is

generic(N :integer := 12); --4 bit pipelined adder is given here as an example

--  Port ( );

end tb_pipelined_adder;

architecture Behavioral of tb_pipelined_adder is

component pipelined_adder is

generic(N :integer := 12); --4 bit pipelined adder (change accordingly)

Port ( A  : in STD_LOGIC_VECTOR (N-1 downto 0);

B  : in STD_LOGIC_VECTOR (N-1 downto 0);

Ci : in STD_LOGIC;

S  : out STD_LOGIC_VECTOR (N downto 0);

--           Co : out STD_LOGIC;

CLK, RSTn : in STD_LOGIC );

end component;

signal A,B : std_logic_vector (N-1 downto 0);

signal Ci  : std_logic := '0';

signal clk : std_logic := '0';

signal rst : std_logic := '1';

signal Sum : std_logic_vector (N downto 0);

--   signal Co  : std_logic;

constant clk_period : time := 70 ns;     -- Clock Frequency

constant  P: integer:= 3;                -- number of pipeline stages

begin

UTT: pipelined_adder generic map(N=>12) port map(A,B,Ci,Sum,clk,rst); --4 bit pipelined adder (change accordingly)

clk_process :process

begin

clk <= '0';

wait for clk_period/2;

clk <= '1';

wait for clk_period/2;

end process;

stim_proc: process

begin

-------------12-bit ADDER INPUTS are given here as an example

rst <= '1';

A <= "000000000000";

B <= "000000000000";

Ci<= '0';

wait for clk_period;

rst <= '0';         --- 27+127

A <= "000000011011";

B <= "000001111111";

Ci<= '0';

wait for (P)*clk_period;

A <= "000000011011"; -- 27+127+1

B <= "000001111111";

Ci<= '1';

wait for (P)*clk_period;

A <= "000000011011"; -- 27+4095

B <= "111111111111";

Ci<= '0';

wait for (P)*clk_period;

A <= "000000011011"; -- 27+4095+1

B <= "111111111111";

Ci<= '1';

wait for (P)*clk_period;

A <= "000001110101"; --117+127

B <= "000001111111";

Ci<= '0';

wait for (P)*clk_period;

A <= "000001110101"; --117+127+1

B <= "000001111111";

Ci<= '1';

wait for (P)*clk_period;

A <= "000001110101"; --117+4095

B <= "111111111111";

Ci<= '0';

wait for (P)*clk_period;

A <= "000001110101"; --117+4095+1

B <= "111111111111";

Ci<= '1';

wait for (P)*clk_period;

std.env.finish;

---------------------------4-bit ADDER INPUT-------------------------------

end process;

end Behavioral;

- Simulation Waveforms:

Case 1 :

Case 2 :

- Post-synthesis Schematic Diagram:

Case 1:

Case 2:

- Hardware Report:

Case 1:

Case 2:

- Compare Hardware & Clock Speed:

Hardware Comparison:

For both case 1 and case 2 they use over 320 FF, but in case 1 it uses 322 registers to make the 2-pipelining work and in case 2 it uses 333 registers in order to make it work. So, in these two cases, case 1 uses less hardware than case 2 uses in order to make the design.

Clock speed Comparison:

The speed than the time the first case takes is about 100ns to go through a full clock cycle, but in case 2 it uses less time at 70ns for a full clock cycle of the design.

- Conclusion:

We learned in this project how to implement pipelining in the Vivado environment as well as comparing two cases of our pipelining. In the first case we designed a circuit using 2 pipelines for 12 1-bit full adders and in the second case we used 3 pipelines for the 12 1-bit full adders. Between the two cases here, case 1 ended up using less hardware than case 2 but it took more time to go though a full clock cycle than case 2 did. This showed us that 3 pipelines is faster than 2 pipelines for this particular project but it is more hardware intensive to design than using less pipelines as seen. Overall, this was a very fun and interesting project as it taught me how to implement pipelining in VHDL and in the Vivado environment as well as compare 2 different instances of pipelining and the effects of using more or less pipes in the same design.